b) Given, here, Timer0 is used. Since WGM02:1, WGM01:0 bit so WGM00:1 bit so it will be in 101, it will operate in the Phase correct PWM mode 5. Since COM0B1:1 bits are set to 10, it will produce a non-inverted PWM signal at port D. Since CS bits are set to 101, the pre-scaler value is 1024. The PWM frequency of Output for the Phase correct PWM Mode is:

System clock frequency, fclk 16 MHz = 16 ×106 Hz

OCR0B = 151

Frequency = fPWM = fclk2*N(OCR0B) = 16×106 2*1024*(151) = 51.73 ~ 52 Hz

Duty cycle for non-inverted PWM.

Duty Cycle for, OCR0B = 255D100  => D = OCR0B255  = 151255 = 0.6 * 100 = 60%

4)

| A | B | D | F | Cin | H | hex |
| --- | --- | --- | --- | --- | --- | --- |
| 010 | 010 | 011 | 011 | 0 | 110 | 49B6h |
| 001 | 100 | 011 | 001 | 1 | 000 |  |
| 110 | 000 | 000 | 011 | 0 | 000 |  |
| 100 | 010 | 011 | 010 | 1 | 000 |  |
| 010 | 100 | 000 | 101 | 0 | 000 |  |

(010 010 011 011 0 110)2  = (49B6)16

b) UBBRN = 2745

f = 16 *10^6

baud rate = (16*10^6)/(2*(2745+1) = 2913.32 = 2914

baud error rate = (2400-2914)/2400 = -21% > +-2%